State of Israel

Ministry of Transport and Road Safety

Department of Motor Vehicles and

Maintenance Services

ISRAELI MANDATORY REQUIREMENTS (IMR) FOR CALENDAR YEAR 2027 ***************************************************** NON-ROAD MOBILE MACHINERY (NRMM) NEW MODELS

Introduction

This Israeli Mandatory Requirements (IMR) is the statutory document for importing Non-Road Mobile Machinery (NRMM) into the State of Israel.

The document includes information and requirement for importing NRMM in compliance with updated standards and regulations.

Import of NRMM must be carried out by an accredited dealer, holding an import license issued by Israeli Ministry of Transport and Road Safety (MOT).

The imported NRMM models shall comply with the applicable European Union legislation, including the regulations, directives, delegated acts, implementing acts and applicable amendments listed in this document. In addition, the NRMM must comply with the Israeli Standards, relevant to the calendar year of import.

The U.S. federal regulations and standards listed in this document will be accepted for NRMM produced and marketed in NAFTA countries only (Canada, Mexico and U.S.), according to the federal law. Waivers granted, according to federal law, for third countries will not be accepted.

The directives and regulations listed in this document will be effective according to the subject and the schedule noted for each directive or regulation.

The accredited dealer must submit the complete IMR document signed at the head office of the manufacturer, by an authorized representative of the manufacturer, and duly attested to by a notary public.

In some cases, and for certain items, the manufacturer will be requested to provide a copy of the certificates from an authorized laboratory.

The MOT reserves the right to update the requirements for NRMM during the calendar year 2027.

The Israeli Mandatory Requirements (IMR) document forms can be downloaded from the ministry's website at https://www.gov.il/he/departments/policies/imr_NRMM_2027.

- Definitions

- Carry-Over and New Model

Carry-over model

Non-Road Mobile Machinery:

- That has passed prototype inspection in Israel;

- For which a registration form was issued in one of the previous seven years.

- For which changes, if made, do not affect the structure and/or the main assembly, and therefore do not require prototype inspection.

New model

Non-Road Mobile Machinery:

- That has not passed prototype inspection in Israel;

- For which registration forms have not been issued.

- Non-Road Mobile Machinery categories

- Construction machinery (EN500)

- Earthmoving machinery (EN474)

- Description of Non-Road Mobile Machinery:

This declaration refers to the Non-Road Mobile Machinery (NRMM) detailed below.

Please include all models imported by your Israeli dealer/representative.

| NRMM Type | Model | Engine Model | Net Power (HP) |
| --- | --- | --- | --- |
| NRMM Type | Model | Engine Model | Net Power (HP) |

| Make/Manufacturer: |  |
| --- | --- |
| Location of assembly plant (City/Country): |  |

- Non-Road Mobile Machinery (NRMM) requirement:

The following declaration refers to NRMM models listed in section 2.

|  | Subject | Directive/Regulations |  | Complies with the Directive and Amendment |  |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Base | Amendment | Yes | No | N/A |  |
| 1 | Emissions from Non-Road Mobile Machinery (NRMM) | (EU) No. 2016/1628 | (EU) 2020/1040; (EU) 2021/1068; (EU) 2022/992 |  |  |  | - The NRMM must meet the most recent requirements with regard to air-polluting emissions specified in regulation (EU) 2016/1628 and the amendments. - The time table for registration of an applicable engine category will be according to the placing on market and engine production date specified in regulation (EU) 2016/1628 and the amendments. - Engines shall hold a valid EU type-approval issued under Regulation (EU) 2016/1628, unless the manufacturer demonstrates that a specific exemption or transitional provision remains legally applicable to the particular engine and machinery. - Applicable EU implementing framework includes Commission Delegated Regulations (EU) 2017/654 and (EU) 2017/655, and Commission Implementing Regulation (EU) 2017/656, including applicable amendments. |
|  |  | OR |  |  |  |  |  |
|  |  | EPA 40 CFR Part 1039 (CI engines); Part 1048 (large SI engines); Part 1054 (small SI engines), together with applicable Parts 1060, 1065 and 1068. |  |  |  |  | - Effective for NRMM produced and marketed in NAFTA countries. - The NRMM must meet the most recent requirements with regard to air-polluting emissions specified in the US regulations. - The applicable emission requirements shall be determined according to the relevant U.S. EPA regulation, engine category, model year/production date and the conditions of the EPA certificate of conformity - References to 40 CFR Parts 89 and 90 shall be accepted only where the manufacturer provides documentary evidence that the engine is legally covered by a specific continuing, replacement-engine, exemption or transitional provision. |
| 2 | Electromagnetic compatibility (EMC) | 2014/30/EU OR 2014/53/EU | (EU) 2019/1326 & (EU) 2021/455 including applicable amendments |  |  |  | EMC not required for 'walk behind compactors'. Directive 2014/53/EU applies where the relevant equipment falls within the Radio Equipment Directive; otherwise, Directive 2014/30/EU applies to EMC. |
|  |  | OR |  |  |  |  |  |
|  |  | ISO 13766 |  |  |  |  | Effective for NRMM produced and marketed in NAFTA countries |
| 3 | Noise emission in the environment by equipment for use outdoors | 2000/14/EC | 2005/88/EC and subsequent amendments, including (EU) 2019/1243, (EU) 2024/1208, (EU) 2024/2839 and (EU) 2024/2749 |  |  |  | - For drill machines - EN 791 standard will be accepted - Noise emissions not required for crushers - NRMM produce in NAFTA countries will be accepted according to manufacturer's requirements |
| 4 | Sound level at operator's position (for equipment with cab) | ISO 6394 |  |  |  |  | Effective for NRMM produced in NAFTA countries. |
|  |  | OR ISO 6396 |  |  |  |  |  |
|  |  | OR ANSI/SAE J1166 |  |  |  |  |  |
| 5 | Machinery | 2006/42/EC (EU) 2023/1230 | including applicable amendments |  |  |  | Machinery placed on the EU market up to 19 January 2027 Machinery placed on the EU market from 20 January 2027 EC type-examination certificates and approval decisions issued under Directive 2006/42/EC remain valid until their expiry in accordance with Article 52(2) of Regulation (EU) 2023/1230. |
|  |  | OR |  |  |  |  |  |
|  |  | ISO 3449 SAE J231 (FOPS) |  |  |  |  | Effective for NRMM produced and marketed in NAFTA countries |
|  |  | And ISO 3471 SAE J 1040 (ROPS) |  |  |  |  |  |
|  |  | And OSHA regulation (standards 29 - CFR) |  |  |  |  |  |
|  |  | And ISO 3450 and ISO 10265 (for crawler machines) and ISO 17063 (for pedestrian-controlled machines) (brakes) |  |  |  |  |  |
| 6 | CE declaration of conformity |  |  |  |  |  | Not required for NRMM produced in NAFTA countries |
| 7 | CE conformity marking | 2006/42/EC (EU) 2023/1230 |  |  |  |  | Directive 2006/42/EC, as applicable until 19 January 2027; Regulation (EU) 2023/1230, as applicable from 20 January 2027; and other applicable Union harmonization legislation CE conformity marking is not required for NRMM produced in NAFTA countries |
| 8 | Operator’s manual | 2006/42/EC OR ISO 6750 OR SAE J 920 | (EU) 2023/1230 |  |  |  | Directive 2006/42/EC applies to machinery placed on the EU market before 20 January 2027; Regulation (EU) 2023/1230 applies from 20 January 2027. |
| 9 | Fuel type unleaded gasoline | Israeli std. 90 |  |  |  |  |  |
| 10 | Fuel type diesel engines | Israeli Std. 107 |  |  |  |  |  |

- Emissions – Certificate of Approval:

According to Regulation (EU) 2016/1628, as amended by Regulations (EU) 2020/1040, (EU) 2021/1068 and (EU) 2022/992, together with the applicable delegated and implementing acts, including (EU) 2017/654, (EU) 2017/655 and (EU) 2017/656, as amended.

| Engine Model | Certificate of Approval No. | Emission level (Stage) |
| --- | --- | --- |

OR

EPA 40 CFR part 1039, 1054 or 1048

| Engine Model | Certificate of Conformity for 2024 Model Year - Certificate No. | Emission Level (Tier) |
| --- | --- | --- |

- Exterior sound according to Directive 2000/14/EC, as amended, including Commission Delegated Regulation (EU) 2024/1208:

A confirmation that the NRMM complies with directive 2000/14/EC must be submitted; therefore, the manufacturer must provide one of the following documents:

EU declaration of conformity or EU type-examination certificate

| document | Provided document |  |
| --- | --- | --- |
|  | yes | no |
| EU declaration of conformity |  |  |
| EU type-examination certificate |  |  |
| EU conformity certificate |  |  |

- Manufacturer's Obligations

For Non-Road Mobile Machinery (NRMM) produced and marketed according to EU legislation and regulations:

- The manufacturer is responsible, through the relevant approval authority, for ensuring all aspects of the approval process and conformity of production (COP).

- The manufacturer is responsible for ensuring that the machinery complies with all applicable EU harmonization legislation, for carrying out or arranging the required conformity-assessment procedure, for preparing the technical documentation, for affixing the CE marking where required, and for issuing the EU Declaration of Conformity.

- The manufacturer shall provide to the M.O.T., upon request, the applicable EC/EU Declaration of Conformity for each NRMM and, where relevant, evidence of engine EU type-approval under Regulation (EU) 2016/1628.

- For engines subject to Regulation (EU) 2016/1628, the type-approval holder is responsible for conformity of production in accordance with the applicable EU type-approval legislation.

- NRMM manufactured and marketed in the United States, Canada or Mexico in accordance with the applicable U.S. Federal requirements:

- The manufacturer is responsible, through the relevant U.S. federal laws, for ensuring all aspects of the approval process and conformity of production.

| - Declaration |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | I hereby declare that the given information is true and correct. |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Name |  |  |  |  |  | Position |  |  |  |  |  |  |
|  | At (Name of Manufacturer) |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Date: |  |  |  |  |  |  |  |  | Signature |  |  |  |
|  | Notary: I the undersigned |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Public notary at |  |  |  |  |  |  |  |  |  |  |  |  |
|  | hereby certify that |  |  |  |  |  |  |  |  |  |  |  | is authorized |
|  | to sign for and on behalf of the manufacturer. |  |  |  |  |  |  |  |  |  |  |  |  |
|  | This person appeared before me and signed on (Date) |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Person’s identity no. / passport no. is |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | I have here into set my seal and signature |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Date |  |  |  |  |  | Signature |  |  |  |  |  |

Note:

If the status as a representative of the manufacturer has not been changed, a declaration from previous years duly witnessed by a notary will be accepted.