State of Israel

Ministry of Transport and Road Safety

Department of Motor Vehicles and

Maintenance Services

ISRAELI MANDATORY REQUIREMENTS (IMR) FOR CALENDAR YEAR 2027 ***************************************************** NON-ROAD MOBILE MACHINERY (NRMM) CARRY-OVER MODELS from CY 2026

Introduction

This Israeli Mandatory Requirements (IMR) is the statutory document for importing Non-Road Mobile Machinery (NRMM) into the State of Israel.

This Israeli Mandatory Requirements document constitutes the statutory requirements for the import of carry-over Non-Road Mobile Machinery into the State of Israel.

The document includes information and requirements for importing NRMM in compliance with updated standards and regulations.

Import of NRMM must be carried out by an accredited dealer, holding an import license issued by Israeli Ministry of Transport and Road Safety (MOT).

The imported NRMM models must comply with the European Union (EU) directives and regulations listed in this document and their latest amendments; In addition, the NRMM must comply with the Israeli Standards, relevant to the calendar year of import.

The U.S. federal regulations and standards listed in this document will be accepted for NRMM manufactured for and legally marketed in the United States, Canada or Mexico under the applicable U.S. Federal requirements. Waivers or exemptions shall be accepted only where documentary evidence demonstrates that they legally apply to the specific engine or machinery.

The directives and regulations listed in this document will be effective according to the subject and the schedule noted for each directive or regulation.

The accredited dealer must submit the complete IMR document signed at the head office of the manufacturer, by an authorized representative of the manufacturer, and duly attested to by a notary public.

In some cases, and for certain items, the manufacturer will be requested to provide a copy of the certificates from an authorized laboratory.

The MOT reserves the right to update the requirements for NRMM during the calendar year 2027.

The Israeli Mandatory Requirements (IMR) document forms can be downloaded from the ministry's website at https://www.gov.il/he/departments/policies/imr_NRMM_2027.

- Definitions

- Carry-Over and New Model

Non-Road Mobile Machinery:

- Carry-over model means an NRMM model for which an Israeli prototype approval or registration specification was issued during one of the preceding seven calendar years and for which no change has been introduced that may affect its regulatory classification, safety, environmental performance, structure, principal assemblies or compliance with the conditions of the original Israeli approval.

- Changes to the engine, emissions system, rated power, braking, steering, operator protective structures, electrical or electronic systems, safety-related software, remote-control functions or intended use shall be declared and may require classification as a new model.

New model

Non-Road Mobile Machinery:

- That has not passed prototype inspection in Israel;

- For which registration forms have not been issued.

- Mobile Construction Machinery categories

- Construction machinery (EN500)

- Earthmoving machinery (EN474)

- Description of Non-Road Mobile Machinery:

This declaration refers to the Non-Road Mobile Machinery (NRMM) detailed below.

Please include all models imported by your Israeli dealer/representative

| NRMM Type | Model | Engine Model | Net Power (HP) |
| --- | --- | --- | --- |

| NRMM Type | Model | Engine Model | Net Power (HP) |
| --- | --- | --- | --- |

| Make/Manufacturer: |  |
| --- | --- |
| Location of assembly plant (City/Country): |  |

- Non-Road Mobile Machinery (NRMM) requirement:

The following declaration refers to NRMM models listed in section 2.

|  | Subject | Directive/Regulations |  | Complies with the Directive and Amendment |  |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Base | Amendment | Yes | No | N/A |  |
| 1 | Emissions from Non-Road Mobile. Machinery (NRMM) | (EU) 2016/1628 | (EU) 2020/1040; (EU) 2021/1068; (EU) 2022/992 |  |  |  | - The NRMM must meet the most recent requirements with regard to air-polluting emissions specified in regulation (EU) 2016/1628 and the amendments. - The time table for registration of an applicable engine category will be according to the placing on market and engine production date specified in regulation (EU) 2016/1628 and the amendments. - An engine approved under Directive 97/68/EC shall be accepted only where the manufacturer demonstrates, with documentary evidence, that a specific exemption, replacement-engine provision or other transitional provision under Regulation (EU) 2016/1628 remains legally applicable to that particular engine and machinery at the relevant placing-on-market date. - Applicable EU implementing framework includes Commission Delegated Regulations (EU) 2017/654 and (EU) 2017/655 and Commission Implementing Regulation (EU) 2017/656, including applicable amendments. Carry-over status of the machinery in Israel does not constitute an exemption from the engine placing-on-market requirements. |
|  |  | OR |  |  |  |  |  |
|  |  | EPA 40 CFR Part 1039 (CI engines); Part 1048 (large SI engines); Part 1054 (small SI engines), together with applicable Parts 1060, 1065 and 1068. |  |  |  |  | Machinery manufactured for and legally marketed in the United States, Canada or Mexico under the applicable U.S. Federal requirements. - The NRMM must meet the most recent requirements with regard to air-polluting emissions specified in the US regulations. - The applicable U.S. emission requirements shall be determined under the relevant EPA regulation according to engine category, model year/production date and the conditions of the EPA certificate of conformity or other legally applicable EPA provision. - References to legacy 40 CFR Parts 89 or 90 shall be accepted only where the manufacturer provides documentary evidence that a specific continuing, replacement-engine, exemption or other legally applicable legacy provision covers the individual engine. |
| 2 | CE declaration of conformity - |  |  |  |  |  | Not required for NRMM produced in NAFTA countries |
| 3 | CE conformity marking | Directive 2006/42/EC Regulation (EC) 765/2008 Regulation (EU) 2023/1230 |  |  |  |  | For machinery subject to EU product legislation, the CE marking shall demonstrate conformity with Directive 2006/42/EC for machinery placed on the EU market before 20 January 2027, or with Regulation (EU) 2023/1230 (including applicable amendments) for machinery placed on the EU market from 20 January 2027, together with any other applicable Union harmonization legislation. CE marking is not required under this EU route for machinery accepted solely under the U.S. Federal alternative. |
| 4 | Fuel type unleaded gasoline | Israeli std. 90 |  |  |  |  |  |
| 5 | Fuel type diesel engines | Israeli Std. 107 |  |  |  |  |  |

- Emissions – Certificate of Approval:

The engine shall comply with Regulation (EU) 2016/1628, as amended by Regulations (EU) 2020/1040, (EU) 2021/1068 and (EU) 2022/992, together with the applicable delegated and implementing acts, including (EU) 2017/654, (EU) 2017/655 and (EU) 2017/656, as amended, according to the engine category, production date and date of placing on the market.

| Engine Model | Certificate of Approval No. | Emission level (Stage) |
| --- | --- | --- |

OR

EPA 40 CFR part 1039, 1054 or 1048

| Engine Model | Certificate of Conformity for 2024 Model Year - Certificate No. | Emission Level (Tier) |
| --- | --- | --- |

- Manufacturer's Obligations

For Non-Road Mobile Machinery (NRMM) produced and marketed according to EU directives and regulations:

The machinery manufacturer shall ensure continued conformity of the carry-over machinery with the applicable EU machinery legislation, the technical documentation and the configuration previously approved in Israel, and shall issue the applicable EC/EU Declaration of Conformity. For engines subject to Regulation (EU) 2016/1628, the engine type-approval holder remains responsible for conformity of production and compliance with the applicable engine type-approval requirements.

For Non-Road Mobile Machinery (NRMM) produced and marketed in NAFTA countries according to federal law:

- For machinery accepted under the U.S. Federal alternative, the manufacturer shall ensure that the engine and machinery remain in the configuration covered by the applicable EPA certificate of conformity, exemption or other legally applicable federal provision, and shall provide supporting evidence to the M.O.T. upon request.

| Declaration |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | I hereby declare that the given information is true and correct. |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Name |  |  |  |  |  | Position |  |  |  |  |  |  |
|  | At (Name of Manufacturer) |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Date: |  |  |  |  |  |  |  |  | Signature |  |  |  |
|  | I the undersigned |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Public notary at |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Hereby certify that |  |  |  |  |  |  |  |  |  |  |  | is authorized |
|  | to sign for and on behalf of the manufacturer. |  |  |  |  |  |  |  |  |  |  |  |  |
|  | This person appeared before me and signed on (Date) |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Person’s identity no. / passport no. is |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | I have here into set my seal and signature |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Date |  |  |  |  |  | Signature |  |  |  |  |  |

Note:

If the status as a representative of the manufacturer has not been changed, a declaration from previous years duly witnessed by a notary will be accepted.